Newmark-Newton-Raphson Algorithm – Nonlinear numerical integration

General nonlinear equation of motion can be represented as:

Mx+Cx+Kx+fx,x,x=qt#1.1

Where M,C,K are linear mass matrix, damping matrix and stiffness matrix, respectively. x is solution to the equation, F is nonlinear force within the system. q(t) is the external excitation.

Assume system response at time step tn is known, then the following equality holds

Mxn+Cxn+Kxn+fxn,xn,xn=qtn#1.2

System response at next time step tn+1 should also satisfy

Mxn+1+Cxn+1+Kxn+1+fxn+1,xn+1,xn+1=qtn+1#1.3

The Newmark method introduces the two fundamental assumptions for the system response between adjacent time steps (tn,tn+1)

xn+1=1βΔt2xn+1-xn-1βΔtxn-12β-1xn#1.4a

xn+1 =δβΔtxn+1-xn+1-δβxn+1-δ2βΔtxn#1.4b

Substituting Eq. (1.4a), (1.4b) into Eq. (1.3) and move Q(tn+1) to LHS, we can get

Rxn+1,xn,xn,xn,tn+1≔Mxn+1+Cxn+1+Kxn+1+fxn+1,xn+1,xn+1-q(tn+1) =0#1.5

Note in Eq. (1.5), only xn+1 are unknown, so it can be regarded as a zero function of xn+1, we want to approach xn+1 s.t. Rxn+1,xn,xn,xn,tn+1=0. Of course, the initial guess of xn+1 is xn+1(0)=xn

Δx=J-1Rxn+1i,xn,xn,xn,tn+1#1.6

where J is Jacobian of R,  J≔∂R∂xn+1  xn+1i

xn+1(i+1)=xn+1(i)+Δx  xn+1i

After k numbers of iteration, we can get xn+1(k) s.t.

Rxn+1k,xn,xn,xn,tn+1<ε#1.7

Once Eq. (1.7) is fulfilled, we have reached xn+1 and we can continue to get xn+2

Section 2 will show the practical implementation on the NNMcont code

Linear case:

Under Newmark scheme, we have assumption:

xi≔xi-1+1-γΔtxi-1≔ xi+γΔtxi#2.1

xi≔xi-1+xi-1Δt+Δt20.5-βxi-1≔ xi+Δt2βxi#2.2

The implementation takes an intermediate step for computation efficiency

For linear system, we have EOM

Mx+Cx+Kx=Qt

For time step i, we can define two new variable xi,xi

xi≔xi-1+1-γΔtxi-1#2.3a

xi≔xi-1+xi-1Δt+Δt20.5-βxi-1 #2.3b

Note that xi,xi can be computed from (i-1) state, which is a known state and can be used to compute xi,xi as

xi=xi+γΔtxi#2.4a

xi=xi+Δt2βxi#2.4b

For i state, the system should satisfy

Mxi+Cxi+Kxi=Qti#2.5

Substitute Eq. (2.4a), (2.4b) into (2.5), we get

Mxi+Cxi+γΔtxi+Kxi+Δt2βxi=Qti

M+CγΔt+KΔt2β≔ Sxi+Cxi+Kxi=Qt

xi=S-1Qti-Cxi-Kxi

#2.6

Where

S≔M+γΔtC+βΔt2K

With obtained xi, we can now compute the xi,xi using Eq. (2.4a), (2.4b)

Nonlinear-Case:

For nonlinear case, the EOM takes form

Mx+Cx+Kx+fnl(x)=qt

At time step i

Mxi+Cxi+Kxi+fnlxi=qti#3.1

Under Newmark scheme, we have:

xi≔xi-1+1-γΔtxi-1≔ xi+γΔtxi#†2.1

xi≔xi-1+xi-1Δt+Δt20.5-βxi-1≔ xi+Δt2βxi#†2.2

Plug Eq. (2.1), (2.2) into (3.1), we get

Mxi+C xi+γΔtxi+Kxi+Δt2βxi+fnlxi=qti

M+CγΔt+KΔt2βxi+C xi+Kxi+fnlxi=qti#3.2

Notice, use Eq. (2.2), we can represent xi as:

xi=xi-xiΔt2β#3.3

Plug Eq. (3.3) into (3.2) we get

M+CγΔt+KΔt2βxi-xiΔt2β+C xi+Kxi+fnlxi=qti

MΔt2β+CγΔtΔt2β+K≔ Stgntxi-xi+C xi+Kxi+fnlxi=qti#3.4

Notice xi and x are known value in Eq. (3.4), (xi,t) is the only independent variable here. Move RHS of Eq. (3.4) to LHS we get

Rxi≔ Stgntxi+Stgntxi+C xi+Kxi+fnlxi-qti=0#3.5

We want to apply Newton-Raphson to Eq. (3.5) to iterate through xi(k) that makes Rxik→0

With 1st order approximation, we can represent

Rxi+Δx=Rxi+Δx∂R∂xi​xi=0#3.6

Use Eq. (3.5), we can compute ∂R∂xi

∂R∂xi​xi=∂∂xiStgntxi+Stgntxi+C xi+Kxi+fnlxi-qti

=Stgnt+∂fnl∂xi ​xi≔J(xi)≔Sxi#3.7

Use Eq. (3.6), (3.7) we can compute

Δxi= -S-1xiRxi#3.8

xi can be corrected iteratively by Eq. (3.9a), notice when we update xi, we need to simultaneously update xi,xi as well to bide with Newmark scheme assumption

xik=xik-1-S-1xik-1Rxik-1#3.9a

xik=xik-1-γΔtβS-1xik-1Rxik-1#3.9b

xik=xik-1-1Δt2βS-1xik-1Rxik-1#3.9c

Supplement: xi=xi-xiΔt2β#†3.3 ∂xi∂xi=1Δt2β→Δxi=ΔxΔt2β#s3.1 xi≔xi+γΔtxi ∂xi∂xi=γΔt∂xi∂xi=γΔtΔt2β→Δx=γΔtβΔx#s3.2

We stop until normRxi(k)<tolerance

We have predefined initial condition

x0,x0,Q0#2.2

Notation convention, for section 2, x,x,Q,f all represents vectors. The subscript xn does not represent nth element, it denotes for vector x evaluated at time t= nΔt, where Δt is time step, let final time be T, and we have N time steps, so Δt=TN

With given initial condition, we can compute initial velocity

x0=M-1Q0-Cx0-Kx0#2.3

The system state (xn+1,xn+1,xn+1) are numerically updated using Newmark-β method

Newmark-β method: Deal with system in form Mu+Cu+fintu=fext(t) It updates the states as following: un+1=un+Δtuγ where uγ≔1-γun+γun+1 γ is a predefined parameter γ∈[0,1] un+1=un+Δt1-γun+γΔtun+1#s2.1 un+1≔un+Δtun+Δt221-2βun+2βun+1#s2.2

xi≔xi-1+1-γΔtxi-1≔ xi+γΔtxi#s1.1

xi≔xi-1+xi-1Δt+Δt20.5-βxi-1≔ xi+Δt2βxi#s1.2

Newmark-β method